Noise in Digital Signals

Sources of Noise

Noise = Disturbances that are unwanted/unrelated in a digital signal e.g(sudden spikes,…)

Random noise vs systematic error

- Random noise is stochastic

- Quantization error

- Sampling jitter

- Thermal effect in electronics

- Interference from other signals

- System noise is deterministic

- Measurement offset

- Non-linear response

- Generally, can be calibrated

Stochastic = randomly determined process in a System

Deterministic = System in which no randomness occurs

Quantization Error

Quantization = maps a sample value to one of the possible discrete values

Quantization Step:

Δ = (Vmax - Vmin)/ 2N

Quantization Error:

Δerror =  Δ / 2

Sampling Jitter

Sampling Jitter = the time between samples varies                                                                            causes = Sampling Amplitude uncertainty

Example: For a sinewave with amplitude A and frequency f Hz, maximum error magnitude is ∆𝑉 = 2𝜋𝑓𝐴∆𝑡

Johnson Noise

Derivation: 𝐴𝑠𝑖𝑛 2𝜋𝑓𝑡 has an instantaneous changing rate of 2𝜋𝑓𝐴𝑐𝑜𝑠 2𝜋𝑓𝑡 . Hence Maximum amplitude change after sampling jitter is 2𝜋𝑓𝐴𝑐𝑜𝑠 2𝜋𝑓𝑡,                 ∆𝑡 ≤ 2𝜋𝑓𝐴∆𝑡

Conductive coupling

Interference

3 sources of interference that could be present simultaneously

- Conductive coupling: noise current caused by a changing voltage in a nearby circuit

- When two parallel wires act like a capacitor

Inductive coupling

Inductive coupling: noise voltage caused by a changing current in a nearby circuit

Two inductors in parallel creates a magnetic field

- Resistive coupling: occurs when high-level signals share a wire with low-level signals

Signal-to-Noise Ratio (SNR)

Expressed in decibel (dB)

- Logarithm used to better quantify very large or small values such as SNR

- SNR = 0 dB Signal & noise have SAME power

- We want to have SNR = 20 dB

Signal Averaging

We See that by increasing M, we get a closer approximation to the true signal.

Noise Margin                                                                                                                      Ideal voltages are rarely attained in practice                                   TTL = transistor-transistor logic                                                               CMOS = Complementary metal–oxide–semiconductor

Noise margins in TTL:

- High-level = 2.7 – 2   = 0.7 V

- Low-level = 0.8 – 0.5 = 0.3 V

Hardware Interfacing

Compatible ->

Compatible = TTL output to TTL input

Incompatible = uncertain response at CMOS gate when TTL is HIGH (can’t use TTL with CMOS)

<- Incompatible